| DOCUMENT N. |  |  |  |
| --- | --- | --- | --- |

| DATE: …./…./….. |  |
| --- | --- |

| TITLE: | Quality Assurance Plan TEMPLATE |
| --- | --- |

| COMPILED |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | date |  | date |  |  |
|  |  | [signature and date] |  |  |  |  |  |
| APPROVED BY (Head of Technical department) |  |  |  |  |  |  |  |
|  | date |  | date |  | date |  | date |
|  | date |  | date |  | date |  | date |
| APPROVED BY (Head of Quality Assurance Department) |  |  |  |  |  |  |  |
|  | date |  | date |  | date |  | date |
| [signature and date] |  |  |  |  |  |  |  |
| APPROVED BY (Accountable Manager) |  |  |  |  |  |  |  |
|  |  |  |  | date |  |  |  |
|  |  | [signature and date] |  |  |  |  |  |

|  |  |  | date |  |  |
| --- | --- | --- | --- | --- | --- |

REVISION HISTORY

| ISSUE | CHANGE DESCRIPTION | ISSUE DATE |
| --- | --- | --- |
| N. PAG. ___________ |  |  |

TABLE OF CONTENTS

1 Scope and management 6 1.1 Indication of contractor 6 1.2 Applicability 6 1.3 QP Approval and Update 6 2 List of acronyms and definitions 7 3 Applicable documents 10 3.1 Contractual documents 10 3.2 Applicable Regulation 12 3.3 Kopter and Leonardo Helicopters documents 12 3.4 List of [Supplier Name] applicable documentation define on this QP. 12 3.5 [Supplier Name] Approval 14 3.6 Exclusion for Design activities in charge of Kopter 14 3.7 List of Sub-tier QP involved in the activities 14 4 Organizational roles and responsibilities, and personnel competence 15 4.1 Personnel definition 15 4.2 Education and Training 16 4.3 Personnel capability 16 5 Planning 17 6 Focal Point 18 6.1 [Supplier Name] Focal Point. 18 6.2 Kopter and Leonardo Helicopters Focal Point. 18 7 Documents Quality and Safety requirements 19 7.1 Issue 19 7.2 Record Keeping 19 8 DO-PO RELATIONSHIP 20 8.1 DO-PO Arrangement 21 8.2 Changes affecting the DO-PO Arrangement 22 8.3 DO-PO Documents Exchange 22 8.4 Collaboration on Occurrences and Safety Management 22 8.5 Parts in PO’s Capability List 22 8.6 Documented information at every stage of technical data approval 23 9 Configuration Management 24 9.1 Change Classification and Kopter involvement 24 9.1.1 Management of Changes to the Product Design before Certification 25 9.1.2 Management of Design Changes after Qualification 25 9.1.3 Approval of a Change to the Product 26 10 Design and development 27 10.1 Design control 27 10.2 Basic data and requirements of Design 27 10.3 Software Quality Assurance 28 10.4 System Equipment List 28 10.5 Preliminary Design Data Set (DDS) 28 10.6 Critical Part 29 10.7 Forging and Casting Design Requirements 30 10.8 Special Processes 30 10.9 Design Review 30 10.10 Design Verification and Validation 31 11 Activities and documentation for qualification of parts 32 11.1 Functional Qualification 32 11.2 Manufacturing Qualification 33 11.3 Specific Kopter Program requirements 34 12 Inspection and Testing 35 13 Components designed by sub-tiers 36 14 Product identification and traceability 37 15 [Supplier name] DDS Approval 38 16 Control of non conforming product 40 16.1 Quality notifications 40 16.2 Concessions 40 16.3 Escape / Quality Alerts 40 16.4 NC Related publications 41 17 Delivery Documentation 42 18 Maintenance and operating information 43 19 Continued Airworthiness 45 20 Corrective and preventive actions 46 21 Quality Audits 47 22 Design process analysis and improvement 48 23 Right of access 49 24 Special processes 50 25 Control of counterfeit articles and obsolence 51 26 Purchasing process 52 27 Production 53 28 Post-delivery support 54 29 Management commitment in reviewing and monitoring measurement and kpi 55 30 Management of the safety of the organisation 55 ANNEX A 56 Focal points in accordance with Chapter 6 56 [Supplier Name] Focal Points. 57 Kopter and Leonardo Helicopters Focal Points. 58 ANNEX B 1 COMPLIANCE CHECKLIST to the QRS-108 requirements 1 ANNEX C 1 APPLICABILITY in accordance with Chapter 1.2 1 (including the updated list of SADD) 1

# Scope and management

This Quality Plan is issued by [Supplier Name] in order to: - Ensure adequate Quality Assurance System implementation, during the development and production phase of the item of the contract, according to requirements of the applicable documentation. - Ensure that these rules and procedures are correctly implemented, also with its sub-suppliers. Contents of this QP are applicable to all activities performed by the [Supplier Name] for the development, design, manufacture and qualification of a part or a system which requirements are described by the Kopter Group AG and Leonardo Helicopters Specification see in § 3.3. Each evolution of this document shall be sent to Kopter for approval.

## Indication of contractor

[Supplier Name and Plan address]

## Applicability

This Quality Plan is applicable for development design, qualification and production of the System(s) and Part Numbers listed in “Annex C” of this plan. [List in Annex C all the P/Ns covered on this Quality Plan: Kopter/LH P/N, Supplier P/N, Description, Procurement Spec/SCD, Sub-tier P/Ns if any. Remark: any Software and Complex Electronic Hardware installed on Articles shall be clearly declared and identified].

## QP Approval and Update

[Supplier Name] takes the responsibility to keep the Quality Plan updated and to provide Kopter SQA point of contact the QP updated for review. [Supplier Name] shall review the QP quarterly to verify the accuracy and updating of the data contained in the QP. [Supplier must include this requirement into its internal company procedures] The QP will clearly highlight the modifications from the already approved one. It should be managed according to the principles adopted for configuration management [Insert Supplier Applicable Procedure Number and Title].

[Describe how the Supplier intends to manage the changes to QP and how to submit to Kopter approval].

| # List of acronyms and definitions |  |  |  |
| --- | --- | --- | --- |
| AR |  | Analysis Report CDR (Critical Design Review) Activity to be performed in order to check the design status at the end of detailed design, before the start of validation activities. |  |
| ATP |  | Acceptance Test Procedure Document describing the procedure to be performed, for each item (S/N) produced, to verify, before delivering, that the item has the required functional characteristics and is not affected by any malfunction. |  |
| ATR |  | Acceptance Test Record List of results of all ATP tests/verification executed in order to check an item (S/N).It is part of delivery documentation. |  |
| CDR |  | Critical Design Review Activity to be performed in order to check the design status at the end of detailed design, before the start of validation activities |  |
| CoC |  | Certificate of Conformity Document issued in order to declare the conformity of a delivered item to its drawing/applicable specification, or the partial conformity when referring to a Concession or Production Permit. |  |
| COMO |  | Coordination Memo Form used in order to exchange information between the Supplier and Kopter. |  |
| CMP |  | Configuration Management Plan |  |
| CPE |  | Chief Project Engineer |  |
| CVE |  | Compliance Verification Engineer |  |
| DDP |  | Declaration of Design and Performance A DDP is the commitment of a manufacturer to deliver a product complying with the defined dimensions and performances, based on qualification tests carried out by the major user or by the manufacturer himself. This document summarise the test/verification results performed and declares the qualification status of the P/N. It can be issued before the end of all qualification activities (PDDP or Preliminary DDP). |  |
| DDS |  | Design Data Set It is the collection of the drawings, specifications and technical information in general provided by the Design Organization (DO) to a Production Organization (PO), sufficient for the development of production data assuring the continued manufacturability of parts in conformity with the project data. |  |
| DO |  | Design Organisation It is composed by all departments directly involved and/or responsible of activities that starting from a technical requirements have the purpose to give compliance of the product. |  |
| EFA |  | Experimental Flight Approval Minimum subset of qualification tests/verification to be performed in order to give a limited flight clearance to a P/N. |  |
| HW |  | Hardware |  |
| MoC |  | Means of Compliance Method to be used in order to demonstrate the compliance. |  |
| PDR |  | Preliminary Design Review Activity to be performed in order to verify the status of the design at the end of the preliminary design phase and before the start of the detailed design. |  |
| PO |  | Production Organisation It is composed by all departments that have the responsibility to purchase and produce new equipment or parts, but have not in charge design and qualification activities. |  |
| POE |  | Production Organisation Exposition |  |
| PS |  | Procurement Specification |  |
| PSAC |  | Plan for SW aspect for certification Document describing the activities to be performed in order to demonstrate the compliance to the RTCA DO178B. |  |
| QP |  | Quality Plan Document describing the activities to be performed in order to demonstrate the compliance to the contract requirements. |  |
| QPl |  | Qualification Plan Document describing part of activities to be performed in order to demonstrate the compliance. QPls are all analysis reports, similarity reports, etc. and Qualification Test Procedures (QTPs). For each QTP shall be provided the relevant QTR. |  |
| QPP |  | Qualification Program Proposal or Plan Document resuming all activities that will be performed in order to demonstrate the compliance to technical requirements; it contains reference to Qualification Test Proposal (QTP) and remaining reports (analysis, similarity, etc.) |  |
| QR |  | Qualification Review Activity to be performed in order to check the qualification status of the design, at least before the industrialisation phase. |  |
| QTP |  | Qualification Test Procedure/Plan Test proposal issued in order to detail tests performed for compliance demonstration and Qualification against a Standard or DDP. In order to reach the qualification of equipment can be provided one QTP, describing all tests, or more QTPs each one describing a subset of tests. |  |
| QTR |  | Qualification Test Report Test report issued in order to detail results of tests performed for compliance demonstration and Qualification against a Standard or DDP. In order to reach the qualification of equipment can be provided one QTR, describing all tests, or more QTRs each one describing a subset of tests. |  |
| TAF TC |  | Test Execution Authorization Type Certificate |  |
| TCDS |  | Type Certificate Data Sheet |  |
| TR |  | Test Report Test report issued to detail results of tests performed for compliance demonstration. In order to reach the qualification of equipment can be provided one TP, describing all tests, or more TPs each one describing a subset of tests. |  |
| SAS |  | SW Accomplishment Summary Document contains the compliance documentation related to the requirements defined in the PSAC. |  |
| SCDD |  | Source Control Drawing for Design Drawing/Specification referred to equipment which are designed by a Supplier. It includes the requirements the Supplier has to comply with in order to design the equipment. |  |
| SCN |  | Specification Change Notice It is the document issued in order to notify, transmit and record modifications to Technical Specifications. |  |
| SMS |  | Safety Management System |  |
| SR |  | Similarity Report |  |
| SW |  | Software |  |
| SWQAP |  | Software Quality Assurance Plan |  |
| VDD |  | Version Description Document Document resuming all the test results performed in order to demonstrate the SW compliance to the Technical Specification requirements and declaring the conformity level. This document is to be issued during the SW developing phase and at the end of the qualification activity. |  |
| # Applicable documents |  |  |  |
| ## Contractual documents [remove N/A docs. or add if applicable see QRS-01 req.] |  |  |  |
|  | Contract Order N° [Insert Number of Contract Order] |  |  |
|  | Agreement Statement of Work N°[Insert Number of Agreement Statement of Work] |  |  |
|  | QRS-01 |  | Quality Requirements for Suppliers Issue ___ [Insert Number of Issue the QP is compliant with] |
|  | QRSK-01 |  | Quality Requirements for AW09 Suppliers |
|  | QRS-100 |  | Digital Manufacturing (DMFG) |
|  | QRS-101 |  | First Article Inspection |
|  | QRS-103 |  | Quality Requirements for Subcontracted Parts and GSE, Stockist of Raw Material, Distributors of Parts |
|  | QRS-104 |  | Special Processes |
|  | QRS-105 |  | Management of LH Equipment and tools |
|  | QRS-107 |  | Management of Non-Conforming Articles |
|  | QRS-108 |  | Supplier Quality Plan |
|  | QRS-110 |  | DO-PO Arrangement |
|  | QRS-115 |  | Requirements for Design & Development Suppliers of Airbone Equipment |
|  | QRS-116 |  | Software Development, Quality Requirements for Suppliers |
|  | QRS-117 |  | Complex Electronic Hardware, Quality Requirements for Suppliers |
|  | QRS-118 |  | Requirements for Laboratories and Manufacturewrs of Non-Airbone Equipment for LH Engineering |
|  | QRS-122 |  | Supplier Component Maintenance / Operating Manuals management |
|  | QRS-130 |  | Flow-down of LH Requirements to Sub-Tier Suppliers |
|  | QRS-145 |  | Maintenance for LH |
|  | All the Leonardo Helicopters and Kopter Quality and Safety Instructions and the forms that are recalled on these instructions with the relevant explanations are available on the Leonardo Helicopters website at the follow address: https://www.leonardo.com/it/suppliers/supplier-portal/helicopters/quality-requirements-for-suppliers/ www.leonardocompany.com |  |  |
| ## Applicable Regulation CR (EU) 748/2012 (Annex I: Part 21) Implementing rules for the airworthiness and environmental certification of aircraft and related products, parts and appliances, as well as for the certification of design and production organizations.. EU Part 21 subpart G – Production Organization approval. EU Part 21 subpart J – Design organization approval. CR (EU) 1321/2014 (Annex II: Part 145) On the continuing airworthiness of aircraft and aeronautical products, parts and appliances, and on the approval of organizations and personnel involved these tasks.. EASA CS 27 – Certification Specification for Small Rotorcraft. EASA CS 29 – Certification Specification for Large Rotorcraft. EN9100 – Aerospace series Quality management systems – Requirements (based on ISO 9001:2000) and quality systems – Model for quality assurance in design, development production installation and servicing (based on ISO 9001:1994).. AQAP 2110 – NATO Quality Assurance Requirements for Design, Development and Production. AQAP 2210 – NATO Supplementary Software Quality Assurance Requirements to AQAP 2110. EMAR 145 – Requirement for maintenance organizations. EMAR 21 subpart G – Military production organization approval for products, parts and appliances. EMAR 21 subpart J – Military design organization approval ID-Title [Remove or Add more lines to list the Regulations applicable to the Supplier’s Approvals and Capabilities that are relevant for the scope addressed in this QP, e.g. 14CFR Part 21 subpart G, CAR SOR/96-433 subpart 61, RBAC 21 subpart G etc.]. |
| ## Kopter and Leonardo Helicopters documents |  |  |  |
| Kopter Technical Specification N°[Insert Number of Kopter Technical Specification] Kopter Source Control Drawing N°[Insert Number of Kopter Source Control Drawing] [If the Quality Plan is issued for system or part numbers applicable for more helicopters, defined by different contracts, please removing the lines and insert the sentence “see Annex C”] |  |  |  |
| ## List of [Supplier Name] applicable documentation define on this QP |  |  |  |
| [Insert Design Manual/Control] |  |  |  |
| [Insert Drafting issue and approval of the documents and drawings procedure] |  |  |  |
| [Insert Analysis Methods Validation procedure] |  |  |  |
| [Insert procedures of Product Development, Design Review & Requirements Management, covering the decisions that defines: - The instructions for operating and maintaining the Supplier’s product, and changes to these instructions. - The design (and changes to it) of the Supplier’s product in matters of: - Functionalities and Criticality. - Forms and Dimensions. - Materials. - Interactions and Context. - Special Processes necessary for manufacturing, storing, transporting and installing. The demonstration of compliance with Kopter’s requirements for the product.] |  |  |  |
| [Insert the procedure for demonstrating that the developed product complies with Kopter’s requirements] |  |  |  |
| [Insert the procedure for Kopter participate in its processes related to the development of Supplier’s products or execution of Supplier’s designing tasks] |  |  |  |
| [Insert the procedure for managing the occurrence reporting] |  |  |  |
| [Insert procedure for Internal Surveillance, Improvement and Monitoring of the Compliance with the Applicable Requirements and the Written Procedures of the Design Organization] |  |  |  |
| [Insert Definition and managing of Critical Parts procedure] |  |  |  |
| [Insert Qualification Process procedure] |  |  |  |
| [Insert HW and SW Configuration Management procedure (s)] |  |  |  |
| [Insert Record Keeping and Long Time Archiving procedure] [Insert Corrective and Preventive Action procedure] [Insert Quality Audit and/or Design System Monitoring procedure] [Insert Control of Measuring Gages and Test Equipment procedure] [Insert First Article Inspection procedure] |  |  |  |
| [Insert Special Process qualification procedure] |  |  |  |
| [Insert Non-conforming Products management procedure] |  |  |  |
| [Insert all Applicable Supplier documents define on this Quality Plan] |  |  |  |
| [Supplier Name] shall inform Kopter Suppliers Quality Assurance about any significant modification to the procedures above, to evaluate possible impacts to the scope and the contents of this Quality Plan. |  |  |  |
| ## [Supplier Name] Approval |  |  |  |
| [Supplier Name] Design Management System fulfils EU-PART 21 requirements. Certificate N° [Insert Number of Approval from EASA]. [Supplier Name] Production Management System fulfils [NAA] requirements. Certificate N° [Insert Number of Certification or Approval from NAA]. [Supplier Name] Design [Development/Management] System fulfils [NAA] requirements. Certificate N° [Insert Number of Certification or Approval from NAA]. |  |  |  |
| [Supplier Name] Quality System fulfils EN/AS/JISQ9100 series requirements Certificate N° [Insert Number of Certification EN/AS/JISQ9100]. |  |  |  |
| [Supplier Name] Quality System fulfils ISO 9001 requirements Certificate N° [Insert Number of Certification ISO 9001]. |  |  |  |
| [Supplier Name] Design Organization and Production Organization operate in accordance with QRSK-01 and QRS-01 Leonardo Helicopters requirements see Leonardo Helicopters Statement of Approval N° [Insert Number of Certification LH ]. |  |  |  |
| [List of the all further Supplier achieved approvals] |  |  |  |
| ## Exclusion for Design activities in charge of Kopter |  |  |  |
| [List any exclusion for Design activities in charge to Kopter, if any. Please, do not delete this paragraph; delete only the text and report the sentence “Not Applicable”] |  |  |  |
| ## List of Sub-tier QP involved in the activities |  |  |  |
| [Insert all Sub-tier QP involved applicable on this Quality Plan, if any. Please, do not delete this paragraph; delete only the text and report the sentence “Not Applicable”] |  |  |  |

# Organizational roles and responsibilities, and personnel competence

## Personnel definition

The [Supplier Name] organisation and responsibility definition is reported in the Quality Manual N° [Insert Supplier Quality Manual Number and Title]. [Supplier Name] has defined a dedicated structure of program management, in order to assure the appropriate control and quality of Design and Development activities. The following paragraphs report the Manager’s names involved in this program: - Program Manager [Person Title, Supplier Program Manager Name] is responsible to manage the program in order to meet the contract. - Product Development Accountable Manager [Person Title, Supplier Program Manager Name] is accountable for: - The decisions in matters of the development of the product (design and instructions), including the changes to the product; - The compliance with Kopter’s requirements for the product; - The continued airworthiness of the product; - The quality and accuracy of all the technical and compliance documentation (e.g. PS, QTP, QTR, TP, TR, ATP, DDPs); - The quality and accuracy of all the provided design activities (e.g. tests, inspections, calibrations, simulations); - The provision of resources (staff, tools, facilities and assets) necessary to design a product or to provide a design service; - The Safety of Design [Development/Management] System; - The Internal Surveillance, Improvement and Monitoring of the Compliance with the Applicable Requirements and the Written Procedures of the Design [Development/Management] System. [In case of discharged responsibilities to fulfil the accountability identify, directly, the names and roles of people assigned to manage these responsibilities, as also the roles assigned to execute these responsibilities (e.g. CVE, DER, UM) and reference to the associated lists of these persons authorised for these roles]. - Manufacturing Engineering Manager [Person Title, Supplier Head of Design Organisation Name] is responsible for - Production Accountable Manager [Person Title, Supplier Accountable Manager Name] is responsible to produce the item in conformity with engineering data. - Certifying Staff [Person Title, Supplier Certifying Staff Name] is responsible to sign the conformity documents (e.g. CoCs, EASA Forms 1 or TAG 8130), Concession and Production Permits - Quality Manager [Person Title, Quality Manager Name] is responsible for the Quality and is the official communication channel with Leonardo Helicopters for what concerns Quality issues. The person ensures that [Supplier Name] has a Quality policy and that the Quality policy is published and correctly implemented within all [Supplier Name] functions. The person takes the ultimate responsibility for all Quality aspects, ensuring the execution and approval of this Quality Plan and FAI. - Quality Control Manager [Person Title, Quality Control Manager Name] is responsible for… - Manufacturing Manager [Person Title, Manufacturing Manager Name] is responsible for… - Safety Manager [Person Title, Safety Manager Name] is responsible for… [Include the organization chart]

## Education and Training

The above personnel comply with the education, training, skills and experience requirements defined in the internal procedure [Insert the Reference, Number/s and title/s] [Refer to the qualification and training of the persons making decisions in matters of the compliance with the Kopters’s DO requirements and the persons making decision for the operation of the Design and Production systems].

## Personnel capability

[Supplier Name] ensures that all the people involved in design activities for Kopter articles ([Supplier Number of people working in technical direction department] people) complies with the requirement of the above procedure.

# Planning

The schedule of the all activities to reach the final qualification is defined in the document [Insert the Reference, Number/s and title/s]. [The Supplier shall describe their capacity to address the requested scope of work. This might be done by providing the following: - Project planning - Work breakdown structure - Resource breakdown structure - Risk Assessment]

# Focal Point

## [Supplier Name] Focal Point.

See Annex “A” for Supplier Focal Point in charged in the program. Any exchange of information between Supplier and Kopter / Leonardo Helicopters shall be coordinated through the Coordination Memo (COMO) document. [Kopter suggest to report in Annex “A” to this QP the list of Supplier personnel involved in the program which have an interface role with Kopter/LH. In the event of personnel changes, this Annex would be reviewed and formally communicated to Kopter/LH and the change would be treated as a minor modification, which does not require Kopter re-approval of the entire QP.]

## Kopter and Leonardo Helicopters Focal Point.

See Annex “A” for Kopter and Leonardo Helicopters Focal Point in charge in the program. Any exchange of information between Supplier and Kopter/Leonardo Helicopters shall be coordinated through the Coordination Memo (COMO) document. [The list with the reference of Kopter/LH personnel involved in the program with roles of interface and approval will be included in the Annex “A” to the QP. This list will be kept updated by Kopter/Leonardo Helicopters Supplier Chain Management]

# Documents Quality and safety requirements

## Issue

Documentation will be issued, approved, changed and managed according to internal procedure See procedure [Insert Supplier Applicable Procedure Number and Title] [Description of how the documentation is issued, approved, changed and managed.]

## Record Keeping

[Supplier Name]is responsible to receive, archive and manage all the Design Data Set (DDS) related to the product and their changes (Drawings, Procedures, Technical Specifications, Reports, Verification and Validation data, Documents...) in agreement with the Supplier procedure [Applicable Procedure Number and Title]. The location selected to keep the records will be equipped with suitable devices, assuring protection against accidental damage due to adverse environmental conditions, fire, loss or robbery. The record keeping must be maintained, in compliance with the Leonardo Helicopters procedure QRS-01, for the whole service life of each unit or of the aircraft on which these units will be installed and at the disposal of Kopter/Leonardo Helicopters and Civil/Military Authorities. This requirement is applicable also toward [Supplier Name] Subcontractors and Sub-tiers.

# DO-PO RELATIONSHIP The relationship between Kopter, Leonardo Helicopters and both the Suppliers’s Production and Design Organisations is necessary to fulfil the obligations from the applicable regulations and the requirements established in the Statement of Work. In this chapter these aspects are discussed.

## DO-PO Arrangement The EU Part 21A.133(d) requires the Supplier’s Production Organisation to ensure satisfactory coordination between production and design, for which an arrangement between the Production and the Design Organization is made. The arrangement establishes the provisions assuring that the design information transfer from Design Organization to the Production Organization, enabling the Production Management System processes to guarantee the continued manufacturability of parts in conformity with the design data from the Design Organisation. The arrangement will be formalized as provided in the QRS-110, before the start of the production activities and with the signature of the Accountable Managers: - [Supplier Name] Production Organisation Accountable Manager. - Leonardo Helicopters Production Organisation Accountable Manager. - [Supplier Name] Product Development Accountable Manager. - Kopter Head of the Design Organisation. This Quality Plan is the guideline for the signature of the Arrangement. NOTE: The Supplier shall inform Kopter/LH of any events that may affect the applicability of the DO-PO Arrangement; the Supplier shall also inform Kopter/LH when the PO approval is suspended or affected by an authority finding similar to a NAA/EASA level 1 finding. [Note 1: Other regulations similar to the EU Part 21 requires an Approved Production Organisation to define procedures for controlling design data and subsequent changes to ensure that only current, correct, and approved data is used; however an Arrangement might not be necessary. The Supplier is responsible for consulting with its non-EASA member state NAA that approved its PO in order to develop with Kopter the adequate arrangement. Please delete all these notes and Adjust the text above with reference to the applicable regulation replacing the reference to the EU Part 21.A.133 (d). Note 2: Production Organisations approved by the US-FAA – A DO-PO arrangement is not applicable to Suppliers with PO having the US FAA as its primary approving authority. The Paragraph 15 of this Quality Plan defines the activities performed between Kopter and FAA Approved PO to deliver parts with the applicable Certificate of Conformity (e.g. FAA TAG 8130-3). Please delete all these notes and the text above and write the sentence “Not Applicable for Supplier with an FAA Approved Production Organisation” Note 3: If the Supplier does not have an Approved Production Organisation, please delete all these notes and the text above and write the sentence “Not Applicable for Supplier without an Approved Production Organisation.”]

## Changes affecting the DO-PO Arrangement [Description of how the Supplier manage DO-PO Arrangement changes with Kopter/LH. If an Arrangement is not applicable per 8.1 replace this chapter by the sentence “Not Applicable.”] ## DO-PO Documents Exchange The contractual and qualification documents will be exchanged through Supplier’s commercial and Kopter/Leonardo Helicopters purchasing departments, as per the procedures not related to those addressed in the chapter 8.1. Technical documents will be exchanged through [Supplier Name] PO’s Engineering department and the Kopter DO, as per the procedures related to those addressed in the chapter 8.1. All the documents shall be properly approved by the appropriate persons of Supplier’s organisation, before the delivery to Kopter/Leonardo Helicopters. The documents between [Supplier Name] and its sub-suppliers/sub-tiers will be exchanged in accordance to the agreements or arrangements required by the Supplier’s procedure [Insert Supplier Applicable Procedure Number and Title]. The procedures related to those addressed in the chapter 8.1 prevails over these agreements or arrangements. NOTE: Change Requests will be exchanged between the Supplier and Kopter Design Organisations and out of the scope of this chapter. [Continue the chapter with description, direct or by reference, of how the documentation is being exchanged between Supplier PO and Kopter/LH ] ## Collaboration on Occurrences and Safety Management [Description of Supplier’s PO processes for collaborating with the Design Organisations (Supplier and Kopter) in the management of Occurrences and other aspects for the Safety Management. If the Supplier does not have an Approved Production Organisation nor processes for the management of Occurrences and other aspects for the Safety Management, please write the sentence “The Supplier’s Production Organisation adopts the procedures defined in the arrangement with Leonardo Helicopter’s Production Organisation in these matters”.] ## Parts in PO’s Capability List [Description of Supplier’s PO process for including parts in the Capability Lists (“Prototype” and “New”, as applicable) and the minimum documentation needed. The list is referred in the DO-PO Arrangement when existent.] ## Documented information at every stage of technical data approval [Description of transition from applicable technical data to approved technical data (that is after SADD issue), including the update of Capability List and issue of EASA Form 1 NEW, and, if requested by Kopter/LH, the re-issue of the EASA Form 1 Prototype to NEW, whether the conditions are met.]

# Configuration Management The Configuration Management concerns to the information that defines: - The product design (e.g. requirements and associated information, dimensions, materials, software codes, functions, limitations, operational or maintenance information etc.). - The identification of a product, its parts and configuration (e.g. part number, serial number, modification number etc.). - The registers of the production of the units of a product and its parts. - The status of this information (approval, compliance with Kopter requirements etc.). The Configuration Management is made under the rules stated in the procedure(s) [Insert Supplier Applicable Procedure Number and title]. Among other aspects, these (this) procedure(s) of Configuration Management state(s) the rules for identifying the records of this information, like numbering the drawings and documents, as also to identify the change classification and their impact on the numbering. This information is generated within the operation of the applicable processes discussed in the chapters of this Quality Plan.

## Change Classification and Kopter involvement Changes are not possible in the information that defines the production of the units of a product and its parts, since they reflect the execution of concluded processes, and any subsequent operations generate additional information and governed within the applicable processes discussed in the chapters of this Quality Plan for its generation and release, in which Kopter’s involvement is discussed. Changes to the information that defined the status of the information within the Scope of Configuration Management is automatically managed within governed within the applicable processes discussed in the chapters of this Quality Plan for its generation and release. Changes in the information that defines the product design and its identification related to this design has Kopter involvement to manage it (classify and approve) considered at “deliverable” unit’s (End item) level. In case of changes to internal parts of the “deliverable” unit the Supplier is responsible for evaluating the relevant impact on the “deliverable” unit to determine if the change shall be submitted to Kopter for approval or for awareness only. The generation of the information resulting from the change in the information discussed in the former paragraph is a result of the execution of the applicable discussed in the chapters of this Quality Plan for its generation and release, which can involve Kopter. The criteria for involvement of Kopter is discussed within the following subchapters, especially for the impact on the Type Certificate of Kopter’s product in which the Supplier’s product is installed or integrated.

### Management of Changes to the Product Design before Certification

Until the Certification of Kopter’s Product is obtained, changes to the Supplier’s Product design and on the information that identify a product, its parts and configuration will be submitted to Kopter for acceptance. Typically changes to the information on fit, form and function, as also or other design characteristics made in compliance with Kopter’s requirements.

### Management of Design Changes after Qualification

Upon the achievement of the Certification of a Kopter’s Product, a Type Certificate is issued, therefore a baseline exists, which to be changed requires Kopter to perform the processes defined in Subpart D of the Part 21. The information that defines a Supplier’s product design, and many of the information that identifies a product, its parts and configuration, are components of the TC, therefore changes to this information is a change to Kopter’s product TC. A change to any of the following data, is considered a change to the Type Certificate (21.A.41): - Type Design (21.A.31) aspects: - Drawings and their lists necessary to identify the configuration (dimensions, forms functions etc.) - Specifications and their lists necessary to identify the configuration (fit, context, interactions, applicability etc.) - Information on materials, processes, methods of manufacture and assembly. - Information that constitutes Approved airworthiness limitation sections of instructions for continued airworthiness (e.g. related part number, serial number or modification number; inspection instructions, thresholds and periods; as also lifespan, etc.). - Information that has any data necessary to allow comparisons with later products for the determination of the airworthiness and, if relevant, the environmental characteristics of later products of the same type (typically this is the information presented in the TCDS of a Kopter’s product). - Other TC aspects: - Information that constitutes operating limitations (as per the applicable points of the Subpart G of the CS 27). - Information that constitutes the instructions for continued airworthiness (21.A.7 and the CS 27.1529). Note 1: More aspects are part of the Type Certificate, but typically are not under influence of the design of a Supplier’s product, therefore are not mentioned.

### Approval of a Change to the Product

All Changes to the Supplier’s product will be communicated to Kopter for classification and approval. For each Changes, the [Supplier Name] will send to the Kopter Design Organisation persons (acc. to reference to the SoW) responsible for the design activity. The provided information is composed by the following: - Engineering change order documents, in the format identified in the Quality Plan and in the DO-PO Arrangement, if applicable. - The drawings relevant to the change and all the documents proposing or testifying the demonstration of compliance to the applicable Technical Specification requirements, applicable airworthiness requirements (CS paragraphs) and environmental protection requirements. - The associated compliance statements, description reports, analytical substantiation reports, safety analysis reports, test plans/test reports etc. Changes cannot be implemented until its approval is communicated by Kopter with signature on the SCN. EXCEPTION: The changes in the table below are pre-classified as Very Minor Changes to the Supplier’s product not requiring further demonstration of compliance. Only these specific changes do not require any Kopter approval before the implementation. - Correction of drawing clerical errors (E.g. Graphical errors; formal errors on quotations or references). - Correction of Drawing Part list clerical errors (E.g. Formal errors; incorrect or superseded recall of materials or standards). - Translation of the data set on a different CAD system keeping technical contents. - Change affecting part / specification identification without change of contents (for example: evolution from MIL to SAE; MIL to NAS etc.). Except for contracted and deliverable P/Ns or other identification information (e.g. referred by an Limitation). - Re-arrangement of drawing tree without altering technical contents (for example moving P/N from an assembly drawing to another one transposing the installation instruction and keeping the technical contents). Without impact on contracted and deliverable P/Ns. In this case [Supplier Name] will send through COMO to the Kopter Design Organisation persons responsible for the design activity. The sent information is made by the following: - Engineering change order document, in the format identified in the Quality Plan and in the DO-DO Arrangement, if applicable. - Possible additional documentation to complete change description. [Make reference to Supplier’s Applicable Procedures Number and title]

| # Design and development |  |
| --- | --- |
| During the planning [Supplier Name] and Kopter DO will determine the involvement of Kopter’s DO in the Design and Development, considering the SoW and relevant arrangements or agreements (e.g. DO-DO arrangement). Design planning is detailed in the procedure [Insert Supplier Applicable Procedure Number and title]. |  |
| ## Design control |  |
| The analysis methods used to design the part have been validated in accordance with the procedure [Insert Supplier Applicable Procedure Number and title]. |  |
| ## Basic data and requirements of Design The requirements for the Supplier’s product are defined by Kopter in the associated SoW, directly or by reference, covering all the aspects of a product (e.g. requirements and associated information, dimensions, materials, software codes, functions, limitations, operational or maintenance information etc.). The requirements are also referred in the Procurement Specification and are managed by the Supplier using the [Insert Supplier Applicable Procedure Number and title] within its product development system. While designing the systems’ level of a product starting in compliance with the Kopter’s requirements, [Supplier Name] will identify in the system its characteristics, as also in its subsystems, parts, components or software that are “Significant” to the compliance. This process triggers in the requirements management derivations and lower level requirements to steer both the development of the system itself or of its subsystems, parts, components or software; generating a specification and sub-specifications that shall be prepared by [Supplier Name] and validated with Kopter Design Organisation persons responsible for the design activity, as also per the SoW. Though in the case of the development of the product consists of designing a component or a single part (not a system), [Supplier Name] will execute the [Insert Supplier Applicable Procedure Number and title] to prepare a specification in compliance with the Kopter’s requirements, and eventual derivations and lower level requirements fulfilled by the component or single part characteristics. The above documentation (drawings and documents) will be prepared according to the procedure/s [Insert Supplier Applicable Procedure Number and title]. |  |
| ## Software Quality Assurance |  |
| If there is an applicable software to equipment that are object of the SoW, the development of the SW by [Supplier Name] occurs as defined in the SW Quality Plan [Insert Supplier SW Quality Plan with Number and title], as also per the SoW. [If the scope of the SoW has not define the development or integration of a software in the product, then please write the following: “The development of a Software or its integration is not a part of the scope of the products addressed in this plan. However Supplier acknowledges its role of supporting Kopter with information regarding its scope of the products and the associated SoW for any efforts for the development or integration of a software in the product.] |  |
| ## System Equipment List |  |
| At the Critical Design Review (CDR) a preliminary list of equipment and items will be agreed between [Supplier Name] and Kopter. This list has been divided in the following: - List of TSO certified equipment identifying its approval, as also the validation by EASA in case of non ETSO. - List of “Significant” equipment (P/N’s deliverable that need a specific activity of validation). - List of “Non-Significant” items (P/N’s that will be qualified like an assembly of a superior assembly) The first two lists are annexed to this Quality Plan [Annex…], with the relevant “Manufacturer”. For “Significant” equipment, an “Equipment Specification” (or equivalent Specification Drawing), list will be prepared and submitted to Kopter for approval. For ETSO or TSO equipment that the approval or validation by EASA is not granted yet the future approval or validation owner and date planned for these events shall be shared accordingly in the list. Significant parts, are those parts/equipment that whose failure can result in either minor or major failure condition and that: - include design features or use of materials that requires special qualification inspections to demonstrate their reliability, - are produced by a manufacturing process that requires complex inspections which, if not performed correctly, can result in significant costs to the Company. Non-Significant parts, are all those parts that are not classified as Critical Parts, Primary Parts or Significant. |  |
| ## Preliminary Design Data Set (DDS) |  |
| At the CDR, a preliminary DDS will be provided by [Supplier Name]to Kopter and keep it updated during project subsequent evolution. |  |
| ## Critical Part |  |
| Critical and Hazardous parts are defined and managed according to requirements of QRSK-01; in addition the following [Supplier Name] procedure will be applied: [Insert Supplier Applicable Procedure Number and title]. For each parts defined “Critical”, according to CS 29.602 and CS 27.602, will be guaranteed the following: - Identification of the “Critical” part and the characteristic that make critical the item, on the technical documentation (DDS); - Traceability of the manufacturing process of each item (by means of a S/N or Batch Number); - Qualification of the manufacturing process; - The work order has to highlight that the item is Critical or Hazardous; - Any special process applied has to be qualified; - Any NDT has to be qualified and the relevant test scheme has to be approved by a third level Inspector; - Definition of “Invariable defined process”, freezing the manufacturing process; - Traceability of the operating life, for parts with limited service life, by means of S/N and relevant Log Card management; - The importance to follow the dedicated instructions shall be highlighted .in the maintenance documents; - Any change to the drawing, manufacturing and inspection method or maintenance documents has to be evaluated in terms of fatigue life of the item and must be approved by Kopter Technical and Quality Department before its implementation; - If a part is upgraded to critical or hazardous part this must be identified with a new P/N. All these requirements are applicable also to the [Supplier Name] subcontractors. List of a Critical and Hazardous Part is contained in the [Annex …….] [If the design do not have Critical Parts please delete only the text and report the sentence: “As per the compliance with the SoW the product does not contain Critical Parts.”] |  |
| ## Forging and Casting Design Requirements |  |
| The Forging and casting will be design in accordance with the procedure [Insert Supplier Applicable Procedure Number and title]. The manufacturing process will to be considered as “Invariable defined process”. When the FAI has been approved, in addition to the provisions discussed in chapter 9.1, every Major modification to the manufacturing process will be send to Kopter/LH for approval before its implementation. Major modifications classification criteria are reported in [Insert Supplier Applicable Procedure Number and title] which does not dismiss or affect in any matter Kopter’s DO classification of a change to the TC and the provisions discussed in chapter 9.1. |  |
| ## Special Processes |  |
| Special processes that are in accordance with international procedures (like MIL, for example) or [Supplier Name] procedure [Insert Supplier Applicable Procedure Number and title] don’t request Kopter formal approval. [Supplier Name] is responsible to assure the conformance of Special Processes to specifications, both in house and with its subcontractors. The parameter of Special Processes will be monitored and the process, equipment and operators will be qualified in accordance with internal procedure and Applicable Technical Specification. The list of special processes used in this program is reported in [Annex ………..]. [Supplier Name] is responsible to inform Kopter for major modifications to the applicable Special Process Specifications. Major modifications classification criteria are reported in [Insert Supplier Applicable Procedure Number and title] which does not dismiss or affect in any matter Kopter’s DO classification of a change to the TC and the provisions discussed in chapter 9.1. [If the Supplier does not use special processes to make parts, please, not delete this paragraph but insert the sentence ”As per compliance with SoW the product does not requires Special Processes”.] |  |
| ## Design Review |  |
| The [Supplier Name] has to perform internal Design Reviews following the applicable procedures [Insert Supplier Applicable Procedure Number and title]. A Design Review is closed when each actions item is closed and the relevant documents and records are released and approved, as appropriate. Formal Design Reviews will be performed with Kopter, according to requirements of QRSK-01; the following Design Reviews will be performed: - PDR; at the end of preliminary design phase. At this time the QP and SWQAP shall be approved. - CDR; is a review at the end of detailed design phase, at this time the drawings are formally issued and the activities related to the demonstration of compliance with Kopter’s requirements are done. Upon the conclusion of the CDR the product design baseline will be frozen and the product release from the Supplier’s manufacturing can start. From this time any modification will be managed in accordance with the chapter 9 Configuration Management. NOTE: Within the detailed design phase other design reviews can be done achieve the progressively conclusion of the necessary elements requires in the CDR, Typically due to the complexity of the Supplier’s product. However these reviews do not dismiss or replace the CDR, its criteria of success and effects. For Activities of design and development of components or systems that contain software, additionally the following Design Reviews will be performed: - SSR; with the purpose to verify the correct interpretation and implementation of Kopter’s requirements within the SW requirements contained in the SRS. - TRR; which is a review done before qualification test starting, with the purpose to verify the final expected outputs confirming integration and test positive execution - DR; which is a review done before any not yet completely qualified SW release anticipated because an internal Kopter activity needed it or before Kopter submission a new DDP revision. - FQR/SCR*; which is a review done whenever the SW will be certified according to RTCA DO178B. [Please declare in these paragraphs if the design does not contain any software or complex hardware] |  |
| ## Design Verification and Validation |  |
| [Supplier Name] will involve Kopter DO in test/analysis as determined by both organizations early in the planning of the project. During development phase, preliminary tests/analysis can be performed in order to verify the adopted design solutions and give a preliminary flight clearance to components. Preliminary validation documentation can be issued. The documentation and the activities issued and performed for validation will be the same described in the following subparagraph for preliminary and final Validation and Certification. Design validation will be performed in agreement with the [Supplier Name] procedure [Insert Supplier Applicable Procedure Number and title] and substantiated at the end of all design and development activities, by issuing the final documentation, according to QRSK-01. Validation conclusion will be resumed by issuing final DDPs for: - Each “Significant” equipment; - All the Systems and Subsystems. |  |

# Activities and documentation for qualification of parts

In order to comply with Kopter and Leonardo Helicopters requirements these activities will be performed and the documentation will be issued according to QRSK-01 and QRS-01.

## Functional Qualification

In addition to the provisions of the chapter 10, the activities presented in this subchapter applies to the Functional Qualification of the Supplier’s product. These activities are performed by the [Supplier Name] according to relevant procedure of [Insert Supplier Applicable Procedure Number and title.] The main relevant requirements for functional qualification, can be resumed as follows: - Testing will be performed using laboratories, test rigs, instruments, etc. Validated by [Supplier Name], or by a Subcontractor that has been validate by [Supplier Name] for the relevant activities, and accepted by Kopter. - All the items used during testing will be “conform” to the relevant drawing; this conformity is demonstrated producing a FAI on the item under test or on an item of the same batch. - Algorithms and tools used for analysis will be qualified. - Any certification test before starting will be authorized by Kopter Compliance Verification Engineer (CVE), using Kopter’s applicable form. Documentation that will be issued in order to give evidence of design/validation/qualification results, will be: - Qualification Plan (QPls) - Analysis Reports (AR) - Similarity Reports (SR) - Qualification Test Procedures (QTP) - Qualification Test Reports (QTR) - Plan for Software Aspect of certification (PSAC)* - Software Verification Plan (SVP)* - Software Test Report STD* - Software Test Result STR* - Software Accomplishment Summary (SAS)* - Version Description Document (VDD)* - Declaration of Design and Performance (DDP) - Test Execution Authorization (TAF) NOTE: Preliminary DDPs will be revised in case of: - Change of the declaration of compliance to the procurement specification status - Change of the limitations -Change of applicable ATP (this also applies to final DDP) All these documents will be signed/approved by [Supplier Name] personnel defined in Annex A and according to relevant procedure of [Insert Supplier Applicable Procedure Number and title.] The same will be applied in case of equipment designed by Subcontractors. Kopter approval is required. * Only if the RTCA DO178b is applicable. [If the designed parties do not contain any software systems, please, remove the section dedicate to software PDR.] [If the Supplier does not perform qualification tests because the component is only a manufacturing, please, remove the section and insert the sentence “Not Applicable”.]

## Manufacturing Qualification

For manufacturing qualification a FAI activity will be performed. First Article Inspections will be performed according to requirements of QRS-101 and [Supplier Name] internal procedure [Insert Supplier Applicable Procedure Number and title.] Prior to perform FAI, a FAI Plan will be submitted to Leonardo Helicopters for approval using the QRS-101 (or EN/AS/JISQ 9102 equivalent) forms for those parts not inserted in the [Supplier Name] capability list. Once received formal approval by POA, FAI activities will start. FAI will be submitted to Leonardo Helicopters for approval, using FAI forms previously approved as per QRS-101, for: - Each “Significant” equipment; - Each Systems and Subsystems. The following FAI steps will be performed during the development phase: - a FAI before the first delivery of a new configuration (P/N), in order to have a control of delivered configuration and relevant manufacturing process, by annexing, as a minimum, the outline drawing, part list and ATR of the unit; - a FAI for each item used for qualification/validation test, in order to be sure that the used item is conform to the relevant design documentation; alternatively, a FAI on a unit with the same P/N and a complete dimensional check of the “Qualification item”. During development phase, preliminary FAIs can be performed and submitted to Kopter and Leonardo Helicopters for approval, before the completion of the “Final” FAI. A revision of the FAI will be performed and submitted to Leonardo Helicopters for approval, in case of: - Additional tests/verifications have been performed, in order to move from a preliminary FAI to another Preliminary FAI or to Final FAI. For Critical/Hazardous parts in case of: - Minor modifications to the “Technical” configuration of the “deliverable” equipment; - Modification to the manufacturing process of the “deliverable” equipment. A FAI will be completely reissued in case of “Major” modification of the equipment. Major modifications classification criteria are reported in [Insert Supplier Applicable Procedure Number and title]. If the article under FAI is in the capability list, the FAI Plan and FAI (and subsequent Delta FAI) will be performed internally and copy of FAIR will be sent to LH Relevant Quality Control.

## Specific Kopter Program requirements [Description of how the supplier intends to manage the process and supplier procedures]

# Inspection and Testing

For each “Significant” equipment and for the subsystems/systems an ATP will be prepared, approved by the Kopter and performed for each item, before the delivery. Significant equipment classification criteria are reported in [Insert Supplier Applicable Procedure Number and title]. This recurrent ATP has the purpose to verify the right functioning of the unit or system. This activities are performed by the Supplier according to relevant procedure of [Insert Supplier Applicable Procedure Number and title].

# Components designed by sub-TIERS

The [Supplier Name] can subcontract part of the design to other qualified subcontractors. In this case the responsibility of the issue of the equipment detailed Drawings, complete “Part Lists”, and all other design documentation is under the design office of the other Company (Subcontractor) responsibility. The [Supplier Name] has the responsibility to transmit Kopter/Leonardo Helicopters requirements to the subcontractors and to verify their implementation. Subcontractors of [Supplier Name] have to comply with these requirements. Traceability of all activities performed for design, development, validation and manufacturing of delivered equipment/parts is maintained. The Design Data Set issued by the Subcontractor will be correctly approved by the [Supplier Name]. The evidence of the approval will be on the minute of the PDR and CDR performed with the sub-Suppliers, on the “Part List” document and on the FAI. These activities are performed by the Supplier according to relevant procedure of [Insert Supplier Applicable Procedure Number and title]. The list of major subcontractors and is reported in [Annex ………] of this Quality Plan. [If the Supplier does not have Sub-tier Suppliers, please do not delete this paragraph; delete only the text and insert the sentence “[Supplier Name] do not Subcontract Design Equipment.”] [The Supplier shall indicate how it is flowing down Kopter/LH requirements to its Suppliers. The Supplier shall indicate the list of all of its Suppliers (Sub-tiers) involved in design and production/inspection activity. Remark: any sub-tier QPs shall be made available to Kopter/LH upon request.]

# PRODUCT identification and traceability

Product will be identified, traced and delivered according to requirements stated in: - Applicable drawing; - Applicable Kopter and Leonardo Helicopters Technical Specification; As a detail, at least, the following info will be reported on each “deliverable” unit: - Name, mark or symbol of the Manufacturer as identified by the applicable design data; - P/N as defined by the Designer of the applicable design data; - S/N of the Manufacturer or Batch Number if the serialization is not required; - Modification status: it is the revision of the applicable detail drawing or Part List. The Modification status (or equivalent) shall be marked in a distinct manner from the P/N; - P/N of the Main Supplier in case that the Designer is different from the Main Supplier, shall be added also the Supplier P/N as responsible of compliance with the Kopter and LH Technical Specification; - Kopter P/N or program P/N as defined by Kopter and LH Technical Specification; - Equipment/part description; - Manufacturing date; - Manufacturing quality stamp; - Identification code of applicable concession/deviation permit (if any).

# [Supplier name] DDS Approval

For European (EASA) Suppliers Kopter approves the up-to-date applicable D.D.S. of the [Supplier Name] signing the dedicated form. The [Supplier Name] design organisation assures the transfer of the up-to data D.D.S, .to its Production organisation in accordance with the internal procedure [Insert Supplier Applicable Procedure Number and title] agreed in the DO-PO Arrangement as request on QRS-110. [Supplier Name] Quality department will perform audits to assure the compliance of this requirement. The [Supplier Name] production organisations is responsible to: - prepare its own manufacturing data in compliance with the applicable design data department - assist Kopter/Leonardo Helicopters in dealing with continuous airworthiness - assist Kopter/Leonardo Helicopters in showing compliance with airworthiness requirements - perform manufacturing activity according to the procedure reported in this Quality Plan Kopter will inform officially the [Supplier Name] that the applicable D.D.S. can be considered as Approved using the Statement of Approved Design Data (SADD) form signed by his Chief Project Engineer (CPE). Once the DDS is approved and the SADD is issued [Supplier Name] will issue an EASA form 1 according to “Approved Design Data” instead of “Applicable Design Data”. For U.S.A. suppliers Kopter approves the up-to-date applicable D.D.S. of the [Supplier Name] signing the dedicated form. The [Supplier Name] design organisation assures the transfer of the up-to data D.D.S, .to its Production organisation in accordance with the internal procedure [Insert Supplier Applicable Procedure Number and title] agreed in the DO-PO Arrangement as request on Leonardo Helicopters QRS-110. [Supplier Name] Quality department will perform audits to assure the compliance of this requirement. The [Supplier Name] production organisations is responsible to: - prepare its own manufacturing data in compliance with the applicable design data department - assist Kopter/Leonardo Helicopters in dealing with continuous airworthiness - assist Kopter/Leonardo Helicopters in showing compliance with airworthiness requirements

- perform manufacturing activity according to the procedure reported in this Quality Plan

Kopter will inform officially the [Supplier Name] that the applicable D.D.S. can be considered Approved using the “COMO form” signed by Chief Project Engineer (CPE).

To satisfy the Kopter/Leonardo Helicopters request of an Airworthiness Certify FAA TAG 8130 -3, U.S.A. Supplier can deliver the FAA TAG only if it has a Production Certificate. Kopter/Leonardo Helicopters Procurement will inform the [Supplier Name] that the parts or the systems have achieved the Airworthiness state.

Kopter/Leonardo Helicopters will send to supplier the “PMA Assist Letter” to allow the [Supplier Name] to begin the PMA certification process with FAA.

At the end of PMA certification process [Supplier Name] will send to Kopter/Leonardo Helicopters a copy of PMA Letter or supplement.

[If you are a European (EASA) Supplier please, delete the part dedicate to USA Supplier. If you are an USA Supplier please, delete the part dedicate to European Supplier]

# Control of non conforming product and safety occurrences

[Supplier Name] manages the non-conforming material in compliance with the requirements contained in the Leonardo Helicopters QRS-107 instruction. [Describe how the supplier intends to manage the non-conforming articles and occurrences (hazards / malfunctioning / failures) in accordance with QRS-01 and QRS-107 requirements, including: Quality Notifications, Concessions, and Escapes/Quality Alerts. Describe how the Supplier intends to manage and submit any Service Bulletins to LH. Describe how the supplier flows-down to Sub-tiers the management of Non-conforming articles and Escapes / Occurrences]

## Quality notifications

In case of a Kopter parts Quality Notification,  [Supplier Name] will manage it opening an investigation report, to track the activities and corrective actions put in place on the product to restore its efficiency.

[Describe, directly or by reference, how the Supplier intends to manage the non-conforming articles in accordance with QRS-107 requirements, particularly the Quality Notifications. Describe how the Supplier flows-down to Sub-tiers the management of Quality Notifications.]

## Concessions

[Supplier Name] will submit a concession request to Kopter and/or Leonardo Helicopters, when the non-conformity is detected during the production activities or at the end of it. Concession request will be submitted to Kopter or Leonardo Helicopters relevant Quality Control for approval.

[Describe, directly or by reference, how the Supplier intends to manage the non-conforming articles in accordance with QRS-107 requirements, particularly the Quality Notifications, Concessions, and Escapes/Quality Alerts. Describe how the Supplier flows-down to Sub-tiers the managements of Concessions.]

## Escape / Quality Alerts

[Supplier Name] will inform Kopter and/or Leonardo Helicopters promptly in case of quality escape/alerts, that might affect the safety, integrity of the product delivered. As indicated in QRS-107 paragraph 5.3, [Supplier Name] will inform Kopter and/or Leonardo Helicopters within maximum 48 hours in case of defects that may have impact on airworthiness or safety.

[Describe, directly or by reference, how the Supplier intends to manage the non-conforming articles in accordance with QRS-107 requirements, particularly the Escapes/Quality Alerts. Describe how the Supplier flow-down to Sub-tiers the management of Concessions.]

## NC Related publications

[Supplier Name] will inform Leonardo Helicopters promptly in case of any publication (e.g. Service Bulletin, Service Letter, etc.) related to non-conforming products or quality escape/alerts, that might affect the safety, integrity of the product delivered. As indicated in QRS-107 paragraph 5.3,

[Supplier Name] will inform Leonardo Helicopters within maximum 24 hours in case of defects that may have impact on airworthiness or safety.

[Describe, directly or by reference, how the Supplier intends to manage and submit any Publication (e.g. Service Bulletin) to Kopter and LH. Describe how the Supplier flows-down to Sub-tiers the management of these Publications.]

# Delivery Documentation

For each product delivery the following documentation will be associated to the product as defined on the purchase order and in agreement with the applicable requirements that are defined on QRSK-01. [Describe how the Supplier manages the delivery documentation in accordance QRSK-01 Requirements.]

# Maintenance and operating information

[Supplier Name] will prepare and deliver to Kopter/Leonardo Helicopters, the following documentation and information: - For first and second level activities, data necessary for corrective and preventive maintenance activities; this data will be approved by the Kopter Engineering Department - For third level activities, the equipment Maintenance Manual, contents of this manual are under [Supplier Name] responsibility - Any information or data necessary for the operation of Kopter’s product in which the Supplier’s product will be installed or integrated. The above documentation will be referenced in the appropriate box of DDP. - First Level (or Organisational Level) The aim of Level 1 maintenance is to keep each aircraft available. This implies, in case of a malfunction, the quick and easy exchange of components and modules, as well as engines replacement. Level 1 activities are expected to include: - Servicing activities - Pre- and post-flight inspection - Functional checks - Trouble shooting - Preventive maintenance - Corrective maintenance (part replacement) - Software loading - Simple modification - Second Level (or Intermediate Level) The aim of Level 2 maintenance is to maintain at the highest possible level of efficiency the aircraft fleet. This implies, mainly, repair of assemblies and subassemblies replaced at Level 1. Level 2 activities are expected to include: - Repairs down to modules and subassemblies - Minor structural repairs to the airframe - Scheduler inspections - Modifications - Technical assistance to the Level 1 organisation - Third Level (or Depot Level) The aim of Level 3 maintenance is to assure the achievement of all repairs and overhaul activities beyond Levels 1 and 2 capabilities. Such activities are generally performed at OEM facilities. Level 3 activities are expected to include: - Repairs down to full reconditioning - Repairs requiring special skills or (test) equipment - Major structural repairs to the airframe - Major scheduler inspections - Modifications and update programmes - Technical assistance to the Level 1 and 2 organisations - Software modifications - Preservation of complete aircraft. The above information is made in conformity with the design of the Supplier’s product using its internal procedures [Insert Supplier Applicable Procedure Number and title.] [If the Supplier item does not require Maintenance Operating information, please not delete this paragraph; delete only the text and insert the sentence “Not Applicable”]

# Continued Airworthiness

[Supplier Name] shall guarantee the “Continued Airworthiness” of its product. This includes managing the changes, airworthiness directives, reported occurrences, alert bulletins, etc. The changes in the Supplier’s product after Kopter’s product Type Certification are managed as discussed in the applicable provisions of the chapters 9 and 10. The other aspects of the Continued Airworthiness of its product are managed as per its internal procedures [Insert Supplier Applicable Procedure Number and title.] After delivery of a product or an information to Kopter and/or Leonardo Helicopters, if [Supplier Name] identifies any defects or threats to the airworthiness or safety of its product, it will inform Kopter and/or Leonardo Helicopters within 48 hours. [Supplier Name] will undertake the appropriate corrective actions after the Kopter indications. [Supplier Name] give Kopter and Leonardo Helicopters all the necessary support for the resolution of problems, as also for occurrence or safety reports.

# Corrective and preventive actions

Corrective and preventive actions procedure [Insert Supplier Applicable Procedure Number and title] will be applied during the period of the contract. A corrective action will be opened when a non-conformity is found in a product/process. The corrective action will contain: - description of the problem/defect; - classification of the problem (major/minor); - definition of the root cause of the non-conformity; - immediate corrective action; - final corrective action; - verification of effectiveness. A preventive action can be required to be opened and managed by a Function responsible, Quality Assurance dept. and/or Accountable Manager, in order to improve the internal Quality Management System for product and/or internal process.

# Quality Audits

Internal quality audit will be performed to technical department according to internal procedure [Insert Supplier Applicable Procedure Number and title]. Quality Assurance Department will plan the internal quality audits. All departments will be planned to be audited once every two years verifying the conformity to the requirements of this Quality Plan. Audit records including audit check lists, audit results and corrective actions will be kept available in case of Kopter/Leonardo Helicopters request. [Supplier Name] is responsible to perform audits at its sub-suppliers. Kopter and Leonardo Helicopters and Authorities can perform audits toward [Supplier Name] departments and subcontractors; in this case Kopter/Leonardo Helicopters inform previously [Supplier Name] Quality Assurance Department in order to agree audit date and details.

# Design process analysis and improvement

[Supplier Name] will facilitate the monitoring and surveillance of the design process made by Kopter, as also will support Kopter efforts with authorities and accreditation agencies. [Supplier Name] will plan and implement monitoring, measurement, analysis and improvement methods particularly related to the design process, establishing suitable metrics representative for the activities performed see procedure [Insert Supplier Applicable Procedure Number and title] that permits to define any corrective action useful to improve the process.

# rIGHT OF Access

Kopter/Leonardo Helicopters have free access to [Supplier Name] that will permit the access to Kopter/Leonardo Helicopters representatives, Kopter/Leonardo Helicopters Customer, Civil and/or Military Authorities accompanied by Kopter/Leonardo Helicopters personnel; furthermore, [Supplier Name] shall guarantee the free access to Subcontractors facilities. The right of access includes access to the information necessary to verify the internal and continued compliance of the Supplier’s organisation (and its Sub-tiers) with the applicable requirements of the associated Statement of Work, the QRSK-01, QRS-01 and the regulations applicable to Kopter and Leonardo Helicopters products, as also to verify the adherence to the procedures of this Quality Plan and referred procedures. If necessary, the Supplier shall produce information considered “confidential”, removing the confidential or exclusive property content, as far as necessary to provide information to the satisfaction of Kopter and Leonardo Helicopters.

# Special processes

[Supplier Name] shall manage special processes according to procedure [Insert Supplier Applicable Procedure Number and title] [Indicate and describe the Supplier Control System and related Procedures in place for special process qualification and control] [Indicate the NDT Responsible Level 3 in Annex A, once notified as per dedicated paragraph about Personnel competence, in QRS-01 main document] [Include the list of Special Processes performed per National/International Specifications, Supplier Proprietary Specifications, Kopter/LH Process Specification, with detail of the Kopter Articles where these special processes are applied and specify Special Processes and sources as well as theirs NADCAP status] [Specify the Subcontracted Special Processes and sources in Annex…..].

# CONTROL OF COUNTERFEIT ARTICLES and obsolence

[Supplier Name] ensures control of counterfeit articles according to procedure [Insert Supplier Applicable Procedure Number and title] [Supplier Name] ensures that pro-active and preventive obsolesce management is implemented, controlled and monitored within the lifecycle of the product [Insert Supplier Applicable Procedure Number and title] [Supplier Name] provides “Obsolescence Notification form” QRS-01_F07 when obsolescence or a potential obsolescence occurs to the following email box: scp.lhd@leonardo.com [Describe how the Suppliers manages counterfeit articles prevention and procedures in place, in accordance with QRSK-01].

# PURCHASING PROCESS

[Supplier Name] shall manage purchasing process according to procedure [Insert Supplier Applicable Procedure Number and title]. [Describe how the procurement process is managed; focus on these items: - Supplier approval and control; - How the Kopter/LH requirements are flowed-down to Sub-tiers; - How the purchased Articles are controlled - How the supplier procures raw material from LH approved source - How special processes are managed through LH approved sources (DQP)]

# PRODUCTION According to QRS-01, QRSK-01 rules and [Supplier Name] QMS procedure for production activities mangement, all the internal production activities are managed with a Work Orders.

[Describe how the Supplier manages the production processes and LH involvement including: - Planning of Product Realization: supplier planning shall comply with requirements defined by LH Manufacturing Engineer - Control of Documentation: reference to supplier internal procedure to manage documentation received and internal flow down - Production documents issuance, change and approval - Production Documentation: the supplier shall have a work order that recalls the steps to be followed - Control of Production Equipment, tools and Software Programs: supplier shall indicate how he keeps under control all the equipment in use with related responsibilities - Product identification, part marking and traceability - Tools and instruments management - Management of Critical parts - Inspection and Testing - Production process control - First Article Inspection and LH involvement - Storage and packaging - Digital Manufacturing, where applicable] - Critical operation: all the critical operation shall be identified with the letter “C” - Implementation of FOD prevention program (see QRS-01)]

# POST-DELIVERY SUPPORT

[The Supplier shall provide assistance to Kopter/LH or its customers upon request within contractual clauses with Kopter/LH, including support and assistance (investigations, etc.) for management of any non-conforming articles and occurrences.]

# management COMMITMENT IN reviewING AND monitoring measurement and kpi

[Describe how the Supplier Management monitors quality and safety objectives. The Supplier shall define internal KPI focused on customer satisfaction. The Supplier shall analyze KPI and take adequate actions for improvement]. # management of the safety The [Supplier Name] shall manage the safety of its organisation according to its [Insert Supplier’s Safety Policy] and its procedures [Insert Supplier Applicable Procedure Number and title.] The management of the safety within the [Supplier Name] organisation includes its Design and Production Organisations. The safety management is made in compliance with the ICAO Annex 19 and with the regulations enforced by the authorities in charge of the approval or surveillance of the [Supplier Name] organisations. The [Supplier Name] Safety Management System is integrated with Leonardo Helicopters’ Safety Management Systems, especially regarding the promotion of the usage by Supplier’s personnel of the voluntary reporting provided by Kopter/Leonardo, as also for the cooperation for the achievement of the safety objectives of these SMS. [The Supplier holding a civil authority approval and having procured part numbers in their capability list shall provide references to their own safety policy and safety procedures. The Supplier without a civil authority approval shall describe how personnel are trained to meet safety requirements (e.g. E-learning available in LH supplier portal). In both cases, the Supplier shall establish a clear and direct communication link with customer focal point(s) in Annex A, formalized through technical agreements within the contract, in order to identify and manage any safety issues by reporting system, in accordance with QRS-01. If the Supplier does not have a Safety Policy, therefore nor a SMS, please replace the content of this chapter with the following: The Supplier is binding to Leonardo Helicopters Safety Policy to manage the safety of its Design Organization, as stated in [Insert Supplier’s Statement of Binding.] The Supplier is in compliance with the QRS-01 to implement a system to manage the safety of its Production Organization, as planned with Leonardo Helicopters.]

ANNEX A

| TITLE: | Focal points in accordance with Chapter 6 |
| --- | --- |

[Supplier Name] Focal Points. Name; Function [example]; Phone, e-mail address. Design Office Director / Program Manager; Phone : e-mail:. Senior Project Manager; Phone : e-mail:. Project Manager; Phone : e-mail:. Purchaser; Phone : e-mail :. Purchaser; Phone : e-mail :. Production Director/ Customer Support; Phone : e-mail:. Quality Manager; Phone : e-mail :. Administrative Director; Phone : e-mail:. Production Manager; Phone : e-mail :.

Current communications are exchanged through coordination memo.

[NOTE: Create a line for each Managerial Role in charge of a Product Development Responsibility for Department, Name and Contact.]

Kopter and Leonardo Helicopters Focal Points. Dept.; Name; Contact. Head of Design Organization; Michele Riccobono; E-mail: michele.riccobono@koptergroup.com Phone:+41 794521533. Responsible Manager LH plant; [….]; E-mail: Phone:. Chief Engineer; [….]; E-mail: Phone:. Chief Project; [….]; E-mail: Phone:. Supplier Quality Assurance; [….]; E-mail: Phone:. Procurement and Supply Chain; [….]; E-mail: Phone:. Quality Concession notified to LH plant to [Name of Supplier]; [….]; E-mail: Phone:.

Current communications are exchanged through Coordination Memo.

ANNEX B

| TITLE: | COMPLIANCE CHECKLIST to the QRS-108 requirements |
| --- | --- |

| SCOPE: | The following checklist shall be used for verifying the correctness and completeness of your QP. Supplier shall declare the compliance, non-compliance or non-applicability for each item of the list, by checking the relevant check-box. In case of deviation from QP template and/or reference to other supplier procedure, please identify the applicable document(s) and paragraph(s) in the 7th column. Use the “NOTES” column for any additional information. The filling of this check-list and its delivery to Kopter are mandatory for QP approval. |
| --- | --- |

Reviewer(s):

| Review Stage | Name | Function | Date | Signature |
| --- | --- | --- | --- | --- |
| 1 | [….] | [….] | [….] | [….] |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |
| 11 |  |  |  |  |
| 12 |  |  |  |  |
| 13 |  |  |  |  |
| 14 |  |  |  |  |
| 15 |  |  |  |  |

Subject Requirement Legend: 	A = AIRWORTHINESS/SAFETY		P = PROGRAM/PROCESS		D = DESIGN

Level Requirement Legend: 		1 = MANDATORY		2 = MAJOR		3 = MINOR

| Item | QRS-108 Template § | Info Required | Subj. | Lev. | Compliance at stage |  |  | QAP actual § | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | Scope and management of the Quality Plan: |  |  | Y | N | N/A |  |  |
| 1.0 | -- | - Is this QAP based on LH Template? | P | 3 |  |  |  |  |  |
| 1.1 | 1.1 | o Indication of contractor (Supplier name and address). | P | 1 |  |  |  |  |  |
| 1.2 | 1.2 | o Applicability (list of all the P/Ns the Quality Assurance Plan is applicable to – Kopter P/N, Supplier P/N, Description, Procurement Spec/SCD). | A D | 1 |  |  |  |  |  |
| 1.3 | 1.3 | o QP approval and update management (how to manage QAP changes and submit them it to Kopter). | P | 2 |  |  |  |  |  |
| 2 | 2 | o List of acronyms and definitions used in the Quality Assurance Plan. | P | 2 |  |  |  |  |  |
|  | 3 | Applicable Documents: |  |  |  |  |  |  |  |
| 3.1 | 3.1 | o Contractual Documents (such as Contract, SoW). | A D | 1 |  |  |  |  |  |
| 3.2 | 3.2 | o Applicable Regulations (EASA, FAA, TCCA, AQAP, etc.). | A D | 1 |  |  |  |  |  |
| 3.3 | 3.3 | o Kopter and LH Documents (Technical Specification, SCD). | A D | 1 |  |  |  |  |  |
| 3.4 | 3.4 | o List of [Supplier Name] applicable documentation defined in the QAP (Quality Manual, Design Manual, internal procedures). | A D | 1 |  |  |  |  |  |
| 3.5 | 3.5 | o Specific applicable Kopter Program documents. | A D | 2 |  |  |  |  |  |
| 3.6 | 3.6 | o [Supplier Name] Approvals held (Civil Certifications, EN series, ISO series). | P | 1 |  |  |  |  |  |
| 3.7 | 3.7 | o Any exclusion for design activities in charge to Kopter. | D | 2 |  |  |  |  |  |
| 3.8 | 3.8 | o List of Sub-Tiers involved in the activity and Sub-tiers QP (if any). | P D | 1 |  |  |  |  |  |
| 4 | 4 | ORGANIZATIONAL ROLES AND RESPONSIBILITIES, AND PERSONNEL COMPETENCE: |  |  |  |  |  |  |  |
| 4.1 | 4.1 | - Personnel definition (indicating at least Program Manager, Product Development Accountable Manager, Technical, Manufacturing Engineering Manager, Production Accountable Manager, Certifying Staff, Quality Manager/Quality Control Manager/ Manufacturing Manager), summarized into Organizational Charts. | D P | 1 |  |  |  |  |  |
| 4.2 | 4.2 | - Reference to the internal procedure related to the education, training, skills and experience for the above mentioned key personnel. | P | 2 |  |  |  |  |  |
| 4.3 | 4.3 | - Description of the capability of people involved into design activities for Kopter programs in terms of required skills and experience. | D P | 3 |  |  |  |  |  |
| 5 | 5 | Planning: documents reporting the schedule of all activities to reach final qualification. The Supplier shall describe their capacity to address the requested scope of work. This might be done by providing the following: |  |  |  |  |  |  |  |
| 5.1 | - | o Project planning | D P | 2 |  |  |  |  |  |
| 5.2 | - | o Work breakdown structure | D P | 2 |  |  |  |  |  |
| 5.3 | - | o Resource breakdown structure and risk assessment | D P | 3 |  |  |  |  |  |
| 6 | 6 | Focal Points: |  |  |  |  |  |  |  |
| 6.1 | 6.1 | o List of [Supplier Name] points (Annex A can be used to indicate the list of Supplier Focal Points). | D P | 1 |  |  |  |  |  |
| 6.2 | 6.2 | o List of Kopter and Leonardo Helicopters Focal Points (Annex A can be used to indicate the list of Kopter and LH Focal Points). | D P | 1 |  |  |  |  |  |
| 7 | 7 | Documents Quality and Safety Requirements: |  |  |  |  |  |  |  |
| 7.1 | 7.1 | o Issue description of how the documentation is issued, approved, changed and managed. | P | 1 |  |  |  |  |  |
| 7.2 | 7.2 | o Record Keeping: defining timing, location and applicability. | P | 1 |  |  |  |  |  |
| 8 | 8 | DO-PO Arrangement: |  |  |  |  |  |  |  |
| 8.1 | 8.1 | - DO-PO Arrangement: general information. | A D | 1 |  |  |  |  |  |
| 8.2 | 8.2 | - Changes affecting the DO-PO Arrangement: description of how the Supplier manage DO-PO Arrangement changes with Kopter/LH. | A D | 1 |  |  |  |  |  |
| 8.3 | 8.3 | - DO-PO Documents Exchange: description of how the documents are going to be exchanged between Supplier and Kopter/LH. | A D | 1 |  |  |  |  |  |
| 8.4 | 8.4 | - Collaboration on Occurrences and Safety Management: description of Supplier’s PO process with the Design Organizations (Supplier and Kopter) in the management of Occurrences and other aspects for the Safety Management. | A D | 1 |  |  |  |  |  |
| 8.5 | 8.5 | - Parts in PO’s Capability List: description of Supplier’s PO process for including parts in the Capability Lists. | A D | 1 |  |  |  |  |  |
| 8.6 | 8.6 | - Concession Approval: description of Supplier procedure for the Production Organization obtaining approval for concession from the Design Organization. | A D | 1 |  |  |  |  |  |
| 8.7 | 8.7 | - Documented Information at every stage of technical data approval: description of transition from applicable technical data to approved technical data. | A D | 1 |  |  |  |  |  |
| 9 | 9 | Configuration Management: |  |  |  |  |  |  |  |
| 9.1 | 9.1 | o Changes and Kopter involvement: describe how the Supplier is intended to inform and manage the changes. 9.1.1 Management of changes to the product design before certification 9.1.2 Management of changes to the product after certification 9.1.3 Approval of a change to the product | A D | 1 |  |  |  |  |  |
| 10 | 10 | Design and Development: |  |  |  |  |  |  |  |
| 10.1 | 10.1 | o Design Control: define the analysis method used to design parts. | D | 2 |  |  |  |  |  |
| 10.2 | 10.2 | o Basic data and requirements of design: managing of requirements for the Supplier’s product. | A D | 1 |  |  |  |  |  |
| 10.3 | 10.3 | o Software Quality Assurance: indicate the Supplier SW Quality Assurance Plan. | A D | 1 |  |  |  |  |  |
| 10.4 | 10.4 | o System Equipment List: list the preliminary list of equipment and items agreed at the CDR. | D P | 2 |  |  |  |  |  |
| 10.5 | 10.5 | o Preliminary Design Data Set (DDS) | D P | 2 |  |  |  |  |  |
| 10.6 | 10.6 | o Critical Part: indicate how the Supplier is intended to manage critical and hazardous parts. | A D | 1 |  |  |  |  |  |
| 10.7 | 10.7 | o Forging and Casting Design Requirements: how the Supplier is intended to manage forging and casting parts. | A D | 1 |  |  |  |  |  |
| 10.8 | 10.8 | o Special Processes: how the Supplier is intended to manage Special Processes. | A D | 1 |  |  |  |  |  |
| 10.9 | 10.9 | o Design Review: indicate the procedure the Supplier follows to perform PDR, CDR, QR, SSR, TRR, DR and FQR/SCR as applicable. | D P | 1 |  |  |  |  |  |
| 10.10 | 10.10 | o Design Verification and Validation: describing how the Supplier is intended to verify and validate design data. | A D | 2 |  |  |  |  |  |
| 11 | 11 | Activities and Documentation for Qualification of Parts: |  |  |  |  |  |  |  |
| 11.1 | 11.1 | o Functional Qualification: the documentation to be issued to provide evidence of design/validation/qualification results is: QAP, AR, SR, QTP, QTR, PSAC, SVP, STD, STR, SAS, VDD, DDP. | A D | 1 |  |  |  |  |  |
| 11.2 | 11.2 | o Manufacturing qualification: the documentation to be issued to provide evidence of manufacturing qualification is FAI, to be performed in accordance with QRSK-01 and QRS-101. | A D | 1 |  |  |  |  |  |
| 11.3 | 11.3 | O Specific Kopter program requirements: description of Supplier process and procedures. | A D | 2 |  |  |  |  |  |
| 12 | 12 | Inspection and Testing: |  |  |  |  |  |  |  |
| 12.1 | - | o An ATP is expected to be prepared and approved by Kopter. | A D | 1 |  |  |  |  |  |
| 13 | 13 | Components designed by Sub-tiers: |  |  |  |  |  |  |  |
| 13.1 | - | o The Supplier shall indicate how he is flowing down Kopter/LH requirements to its Suppliers. The Supplier shall indicate the list of all of its Suppliers involved in design activity. | A D | 1 |  |  |  |  |  |
| 14 | 14 | Product identification and traceability: |  |  |  |  |  |  |  |
| 14.1 | - | o Products will be identified, traced and delivered according to the requirements stated in: Applicable Drawings, Applicable Kopter Technical Specification. The Supplies shall report on each deliverable unit: | A | 1 |  |  |  |  |  |
| 14.2 | - | - Name, mark or symbol of the Manufacturer - P/N as defined by the Designer - S/N of the Manufacturer or Batch Number - Modification status | A | 1 |  |  |  |  |  |
| 14.3 | - | - P/N of the Main Supplier - Kopter P/N or program P/N - Equipment/part description - Manufacturing date - Manufacturing quality stamp - Identification code of applicable concession/deviation permit (if any) | A | 1 |  |  |  |  |  |
| 15 | 15 | Supplier DDS Approval: |  |  |  |  |  |  |  |
| 15.1 | - | o Describe how DDS are submitted to Kopter for approval and how the Supplier gets aware of DDS approval | A D | 1 |  |  |  |  |  |
| 16 | 16 | Control on Non-conforming Product: |  |  |  |  |  |  |  |
| 16.1 | - | o Description of how the Supplier will manage non-conforming products in accordance with QRSK-01 and QRS-107 requirements. | A D | 1 |  |  |  |  |  |
| 16.1 | 16.1 | o Quality notification: description how the Supplier manage the Non-conforming articles. | A D | 1 |  |  |  |  |  |
| 16.2 | 16.2 | o Concessions: description how the Supplier manage the Concessions. | A D | 1 |  |  |  |  |  |
| 16.3 | 16.3 | o Escape / Quality Alerts: description how the Supplier manage the Escapers / Quality Alerts. | A D | 1 |  |  |  |  |  |
| 16.4 | 16.4 | o NC related Publications: description how the Supplier manage and submit any Publication to Kopter and LH. | A D | 1 |  |  |  |  |  |
| 17 | 17 | Delivery Documentation: |  |  |  |  |  |  |  |
| 17.1 | - | o Description of how the Supplier will manage delivery documentation in accordance with QRSK-01 and QRS-01 requirements. | P | 1 |  |  |  |  |  |
| 18 | 18 | Maintenance and operating information: |  |  |  |  |  |  |  |
| 18.1 | - | o Description of how the Supplier will manage the Component Maintenance Manual, in accordance with QRSK-01 requirements. | A | 1 |  |  |  |  |  |
| 19 | 19 | Continued Airworthiness: |  |  |  |  |  |  |  |
| 19.1 | - | o The Supplier shall explain how he is going to manage any defect in order to ensure to inform Kopter/LH within 48 hours from the discovery of the defect. | A | 1 |  |  |  |  |  |
| 20 | 20 | Corrective and Preventive Actions: |  |  |  |  |  |  |  |
| 20.1 | - | o The Supplier shall describe how preventive and corrective actions are managed. | P | 2 |  |  |  |  |  |
| 21 | 21 | Quality Audits: |  |  |  |  |  |  |  |
| 22.1 | - | o Explain how the Supplier shall monitor Kopter/LH requirements by planning and executing internal and external surveillance audits using a risk-based criteria. | P | 2 |  |  |  |  |  |
| 22 | 22 | Design Process analysis and improvement: |  |  |  |  |  |  |  |
| 22.1 | - | o The Supplier shall plan and implement monitoring, measurement, analysis and improvement methods particularly related to the design process. | P | 1 |  |  |  |  |  |
| 23 | 23 | Right of Access: [MANDATORY] |  |  |  |  |  |  |  |
| 23.1 | - | o “Kopter and Leonardo Helicopters has access to Supplier that will permit the access to Kopter and Leonardo Helicopters representatives, Kopter and LH Customers, Civil and/or Military Authorities accompanied by Kopter and Leonardo Helicopters personnel, furthermore, the Supplier shall guarantee the access to Subcontractor facilities.” | P | 1 |  |  |  |  |  |
| 24 | 24 | Special Processes: |  |  |  |  |  |  |  |
| 24.1 | - | o Describe the Supplier control system and the procedures for Special Process qualification and control (if any). | P | 1 |  |  |  |  |  |
| 24.2 | - | o Indicate the NDT Responsible Level 3 in Annex A. | P | 1 |  |  |  |  |  |
| 24.3 | - | o Include the list of Special Processes performed per National/International Specifications, Supplier Proprietary Specifications, Kopter/LH Process Specifications, with detail of the Kopter articles where these Special Processes are applied. | P | 1 |  |  |  |  |  |
| 24.4 | - | o Specify subcontracted Special Processes and sources. | P | 1 |  |  |  |  |  |
| 25 | 25 | Control of counterfeit articles and obsolence: |  |  |  |  |  |  |  |
| 25.1 | - | o Describe how the Supplier manages counterfeit article prevention. | P | 1 |  |  |  |  |  |
| 26 | 26 | Purchasing Processes: Describe how Supplier procurement process is managed |  |  |  |  |  |  |  |
| 26.1 | - | o Sub tiers approval and control process | P | 2 |  |  |  |  |  |
| 26.2 | - | o Flow-down of Kopter/LH requirements | A | 2 |  |  |  |  |  |
| 26.3 | - | o Control of purchased articles | A | 1 |  |  |  |  |  |
| 27 | 27 | Production: Describe how Supplier manages the production process: |  |  |  |  |  |  |  |
| 27.1 | - | o Production documents (issue, change, approval) | P | 2 |  |  |  |  |  |
| 27.2 | - | o Product identification, part marking and traceability | P | 1 |  |  |  |  |  |
| 27.3 | - | o Tools and Instrument management | A | 1 |  |  |  |  |  |
| 27.4 | - | o Management of critical parts | P | 1 |  |  |  |  |  |
| 27.5 | - | o Inspection and testing | P | 1 |  |  |  |  |  |
| 27.6 | - | o Production process control | P | 2 |  |  |  |  |  |
| 27.7 | - | o First Article Inspection /LH involvement | P | 2 |  |  |  |  |  |
| 27.8 | - | O Storage and packaging | P | 2 |  |  |  |  |  |
| 27.9 | - | o Digital Manufacturing (if applicable) | P | 1 |  |  |  |  |  |
| 27.10 | - | o Implementation of FOD prevention program | P | 1 |  |  |  |  |  |
| 28 | 28 | Post-Delivery support : |  |  |  |  |  |  |  |
| 28.1 | - | o Describe how the Supplier provides assistance to Kopter/LH or to its Customers. | P | 2 |  |  |  |  |  |
| 29 | 29 | Management commitment in reviewing and monitoring measurement and KPI |  |  |  |  |  |  |  |
| 29.1 | - | - Describe how the Supplier monitors quality objectives. | P | 2 |  |  |  |  |  |
| 30 | 30 | Management of the safety of the organization |  |  |  |  |  |  |  |
| 30.1 | - | - Describe how the Supplier manage the safety of its organization. | A | 1 |  |  |  |  |  |
|  |  | ANNEXES: |  |  |  |  |  |  |  |
| a | A | o Annex A: Focal Points in accordance with Chapter 6 – List of all Supplier, Kopter and LH Focal Points. | D P | 2 |  |  |  |  |  |
| b | B | o Annex B: Check-list to QRS-108 requirements. | P | 2 |  |  |  |  |  |
| c | C | o Annex C: Capability List with all P/N covered by the QAP, including: - related PMA/E-TSO/STC, if any - related SADD n° - info if released with EASA/EMAR Form1 NEW/PROTOTYPE or equivalent - info if released with Certificate of Conformity | D P | 2 |  |  |  |  |  |

ANNEX C

| TITLE | APPLICABILITY in accordance with Chapter 1.2 (including the updated list of SADD) |
| --- | --- |

| Program AWXXX [Insert the Helicopter Name “ex. AW09] |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Kopter P/N / Supplier P/N | Designation / Title | Procurement Specification | Contract / Purchase Order | SADD number | Product Approval [Specify what kind of approval is related with a X] |  |  | Delivery documentation [Specify what kind of document is issued with a X] |  |  |
|  |  |  |  |  | PMA | (E)TSO | STC | EASA Form 1 (or equivalent) | EASA Form 1 Prototype (or equivalent) | CoC |